MIDTERM  EXAMINATION

Spring 2022

CS302- Digital Logic Design (Session - 1)

Part 1  2007 to 2022  305 MCQ This File

میری پوری کوشش ہے کہ ہمارے چینل کا کوئی بھی سٹوڈینٹ ناکام نہ ہو ۔

پرچہ پڑھنے سے پہلے آپ اپنے چینل کو سبسکرائب کردیں تاکہ آپ کو میری امتحانات سے ریلٹیٹ تمام ویڈیوزآپ تک پونچھتی رہیں ۔

میری دعا ہے جن لوگوں نے اپنے چینل کو سبسکرائب کیا ہے اللہ تعالیٰ ان کو دُنیا اور آخرت کے امتحان میں کامیاب فرمائے                     آمین

ویڈیو کو زیادہ سے زیادہ شیئر کریں ۔آپ کی ایک شیئر کی گئی ویڈیو کسی ایک بچے کو کامیابی کی طرف لے جا سکتی ہے ۔۔۔۔۔۔۔۔۔۔۔۔۔۔۔۔۔۔۔۔۔۔۔۔۔۔۔۔۔۔.    شکريہ

- A SOP expression is equal to 1 ______________

► All the variables in domain of expression are present

► At least one variable in domain of expression is present.

► When one or more product terms in the expression are equal to 0.

► When one or more product terms in the expression are equal to 1

- The output A < B is set to 1 when the input combinations is __________

► A=10, B=01

► A=11, B=01

► A=01, B=01

► A=01, B=10

- Two 2-bit comparator circuits can be connected to form single 4-bit comparator

► True

► False

- High level Noise Margins (VNH) of CMOS 5 volt series circuits is

_____________

► 0.3 V

► 0.5 V

► 0.9 V (

► 3.3 V

- If we multiply “723” and “34” by representing them in floating point notation i.e. by first, converting them in

floating point representation and then multiplying them, the value of mantissa of result will be ________

► 24.582 (But not sure)

► 2.4582

► 24582

► 0.24582

- The output of the expression F=A+B+C will be Logic ________ when A=0, B=1, C=1. the symbol‟+‟ here represents OR Gate.

► Undefined

► One

► Zero

► 10 (binary)

- If an active-HIGH S-R latch has a 0 on the S input and a 1 on the R input and then the R input goes to 0, the latch will be ________.

► SET (

► RESET

► Clear

► Invalid

- 3.3 v CMOS series is characterized by __________ and _________as compared to the 5 v CMOS series.

► Low switching speeds, high power dissipation

► Fast switching speeds, high power dissipation

► Fast switching speeds, very low power dissipation

► Low switching speeds, very low power dissipation

- The binary value “1010110” is equivalent to decimal __________

► 86 (According to Formula)

► 87

► 88

► 89

- The _______ Encoder is used as a keypad encoder.

► 2-to-8 encoder

► 4-to-16 encoder

► BCD-to-Decimal

► Decimal-to-BCD Priority

- "Sum-of-Weights" method is used __________

► to convert from one number system to other (

► to encode data

► to decode data

► to convert from serial to parralel data

- The maximum number that can be represented using unsigned octal system is _______

► 1

► 7

► 9

► 16

- If we add “723” and “134” by representing them in floating point notation i.e. by first, converting them in

floating point representation and then adding them, the value of exponent of result will be ________

► 0

► 1

► 2

► 3

- The diagram given below represents __________

► Demorgans law

► Associative law

► Product of sum form (According to rule of theorem)

► Sum of product form

- For a 3-to-8 decoder how many 2-to-4 decoders will be required?

► 2

► 1

► 3

► 4

- GAL is an acronym for ________.

► Giant Array Logic

► General Array Logic

► Generic Array Logic

► Generic Analysis Logic

- The Quad Multiplexer has _____ outputs

► 4

► 8

► 12

► 16

- A.(B.C) = (A.B).C is an expression of __________

► Demorgan‟s Law

► Distributive Law

► Commutative Law

► Associative Law (

- 2's complement of any binary number can be calculated by

► adding 1's complement twice

► adding 1 to 1's complement (

► subtracting 1 from 1's complement.

► calculating 1's complement and inverting Most significant bit

- The binary value “1010110” is equivalent to decimal __________

► 86 (According to formula)

► 87

► 88

► 89

- Tri-State Buffer is basically a/an _________ gate.

► AND

► OR

► NOT

► XOR (

- The binary value “11011” is equivalent to __________

►1B (According to rule)

►1C

►1D

►1E

- An important application of AND Gate is its use in counter circuit ►True (

►False

- The OR Gate performs a Boolean _______ function

►Addition

►Subtraction

►Multiplication

►Division

- TTL based devices work with a dc supply of ____ Volts

►+10

►+5

►+3

►3.3

- A standard POS form has __________ terms that have all the variables in the domain of the expression.

►Sum

►Product

►Min

►Composite

- A SOP expression having a domain of 3 variables will have a truth table having ____ combinations of inputs and corresponding output values.

►2

►4

►8 (According to rule)

►16

- A BCD to 7-Segment decoder has

►3 inputs and 7 outputs

►4 inputs and 7 outputs)

►7 inputs and 3 outputs

► inputs and 4 outputs

- In the Karnaugh map shown above, which of the loops shown represents a legal grouping?

►A

► B

► C

►D

- The binary value of 1010 is converted to the product term

►True

►False

- The binary numbers A = 1100 and B = 1001 are applied to the inputs of a comparator. What are the output levels?

►A > B = 1, A < B = 0, A < B = 1

►A > B = 0, A < B = 1, A = B = 0

►A > B = 1, A < B = 0, A = B = 0

►A > B = 0, A < B = 1, A = B = 1

- If an active-HIGH S-R latch has a 0 on the S input and a 1 on the R input and then the R input goes to 0, the latch will be ________.

►SET ()

►RESET

►Clear

►Invalid

- Consider a circuit consisting of two consecutive NOT gates, the entire circuit belongs to a CMOS

5 Volt series, if certain voltage is applied on the input, the output voltage of Logic high signal

(VoH) will be in the range of _______ volts.

►4 to 4.5

►4.5 to 5

►0 to 4.5

►0 to 3.5

- A.(B.C) = (A.B).C is an expression of __________

►Demorgan‟s Law

►Distributive Law

►Commutative Law

►Associative Law

- The 4-bit 2’s complement representation of “+5” is _____________

►1010

►1110

►1011

►0101

- Which of the number is not a representative of hexadecimal system

►1234

►ABCD

►1001

► 1

► 7

► 9

► 16

- The 3-variable Karnaugh Map (K-Map) has _______ cells for min or max terms

► 4

► 8

► 12

► 16

- The binary numbers A = 1100 and B = 1001 are applied to the inputs of a comparator. What are the output levels?

► A > B = 1, A < B = 0, A < B = 1

► A > B = 0, A < B = 1, A = B = 0

► A > B = 1, A < B = 0, A = B = 0

► A > B = 0, A < B = 1, A = B = 1

- A particular Full Adder has

► 3 inputs and 2 output

► 3 inputs and 3 output

► 2 inputs and 3 output

► 2 inputs and 2 output

- The function to be performed by the processor is selected by set of inputs known as ____

► Function Select Inputs

► MicroOperation selectors

► OPCODE Selectors

► None of given option

- For a 3-to-8 decoder how many 2-to-4 decoders will be required?

► 2 (

► 1

► 3

► 4

- GAL is an acronym for ________.

► Giant Array Logic

► General Array Logic (

► Generic Array Logic

► Generic Analysis Logic

- The Quad Multiplexer has _____ outputs

► 4

► 8

► 12

► 16

- A.(B.C) = (A.B).C is an expression of __________

► Demorgan‟s Law

► Distributive Law

► Commutative Law

► Associative Law (

- 2's complement of any binary number can be calculated by

► adding 1's complement twice

► adding 1 to 1's complement

► subtracting 1 from 1's complement.

► calculating 1's complement and inverting Most significant bit

- The binary value “1010110” is equivalent to decimal __________

► 86 (According to formula)

► 87

► 88

► 89

- Tri-State Buffer is basically a/an _________ gate.

► AND

► OR

► NOT

► XOR

- GALcan be reprogrammed because instead of fuses __________logic is used in it

► E2CMOS

► TTL

► CMOS+

► None of the given options

- The device shown here is most likely a

► Comparator

► Multiplexer click here for detail

► Demultiplexer

► Parity generator

- If“1110” is applied at the input of BCD-to-Decimal decoder which output pin will be activated:

► 2nd

► 4th

► 14th

► No output wire will be activated

- Half-Adder Logic circuit contains 2 XOR Gates

► True

► False

- A particular Full Adder has

► 3 inputs and 2 output

► 3 inputs and 3 output

► 2 inputs and 3 output

► 2 inputs and 2 output

- Sum A B C  CarryOut C(A B) AB are the Sum and CarryOut expression of

► Half Adder

► Full Adder

► 3-bit parralel adder

► MSI adder cicuit

- A Karnaugh map is similar to a truth table because it presents all the possible values of input variables and the resulting output of each value.

► True click here for detail

► False

- The output A < B is set to 1 when the input combinations is

► A=10, B=01

► A=11, B=01

► A=01, B=01

► A=01, B=10

- The 4-variable Karnaugh Map (K-Map) has ______cells for min or max terms

► 4

► 8

► 12

► 16

- Generally, the Power dissipation of devices remains constant throughout their operation.

► TTL

► CMOS 3.5 series

► CMOS 5 Series

► Power dissipation of all circuits increases with time.

- The ecimal “8” is represented as using Gray-Code.

► 0011

► 1100

► 1000

► 1010

- (A+B).(A+C) =

► B+C

► A+BC

► AB+C

► AC+B

- A.(B+ C) = A.B + A.C is the expression of

► Demorgan‟s Law

► Commutative Law

► Distributive Law

► Associative Law

- NOR Gate can be used to perform the operation of AND, OR and NOT Gate

► FALSE

► TRUE

- In ANSI/IEEE Standard 754 “Mantissa” is represented by 32-bits bits

► 8-bits

► 16-bits

► 32-bits

► 64-bits

- Caveman number system is Base _5 number system

► 2

► 5

► 10

► 16

- According to Demorgan‟s theorem:

_________

► A.B.C

►

► (Page 74)

►

- The Extended ASCII Code (American Standard Code for Information

Interchange) is a _____ code

\► 2-bit

► 7-bit

► 8-bit

► 16-bit

- The AND Gate performs a logical __________function

► Addition

► Subtraction

► Multiplication

► Division

- NOR gate is formed by connecting _________

► OR Gate and then NOT Gate

► NOT Gate and then OR Gate

► AND Gate and then OR Gate

► OR Gate and then AND Gate

- Generally, the Power dissipation of _______ devices remains constant throughout their operation.

► TTL

► CMOS 3.5 series

► CMOS 5 Series

► Power dissipation of all circuits increases with time.

- Two 2-bit comparator circuits can be connected to form single 4-bit comparator

► True

► False

- When the control line in tri-state buffer is high the buffer operates like a

________ gate

► AND

► OR

► NOT (

► XOR

- The GAL22V10 has ____ inputs

► 22

► 10

► 44

► 20

- The ABEL symbol for “OR” operation is

► !

► &

► #

► $

- The OLMC of the GAL16V8 is _______ to the OLMC of the

GAL22V10

► Similar

► Different

► Similar with some enhancements

► Depends on the type of PALs input size

- All the ABEL equations must end with ________

► “ . “ (a dot)

► “ $ “ (a dollar symbol)

► “ ; “ (a semicolon)

► “ endl “ (keyword “endl”)

- The Quad Multiplexer has _____ outputs

► 4 (

► 8

► 12

► 16

- "Sum-of-Weights" method is used __________

► to convert from one number system to other)

► to encode data

► to decode data

► to convert from serial to parralel data

- A latch has _____ stable states

► One

► Two)

► Three

► Four

- Sequential circuits have storage elements

► True

► False

- The ABEL symbol for “XOR” operation is

► $

► # ► ! ► &

- A Demultiplexer is not available commercially.

► True

► False

- Using multiplexer as parallel to serial converter requires ___________ connected to the multiplexer

► A parallel to serial converter circuit

► A counter circuit

► A BCD to Decimal decoder

► A 2-to-8 bit decoder

- The device shown here is most likely a

► Comparator

► Multiplexer click here for detail

► Demultiplexer

► Parity generator

- The main use of the Multiplexer is to

► Select data from multiple sources and to route it to a single Destination (Page 167)

► Select data from Single source and to route it to a multiple Destinations

► Select data from Single source and to route to single destination

► Select data from multiple sources and to route to multiple destinations

- A logic circuit with an output consists of ________.

► two AND gates, two OR gates, two inverters

► three AND gates, two OR gates, one inverter

► two AND gates, one OR gate, two inverters

► two AND gates, one OR gate

- The binary value of 1010 is converted to the product term

► True

► False

- The 3-variable Karnaugh Map (K-Map) has _______ cells for min or max terms

► 4

► 8

► 12

► 16

- Following is standard POS expression

► True (According to logic)

► False

- The output of the expression F=A+B+C will be Logic ________ when A=0, B=1, C=1. the symbol‟+‟ here represents OR Gate.

► Undefined

► One

► Zero

► 10 (binary)

- The Extended ASCII Code (American Standard Code for Information

Interchange) is a _____ code

► 2-bit

► 7-bit

► 8-bit

► 16-bit

- The diagram given below represents __________

► Demorgans law

► Associative law

► Product of sum form (According to rule)

► Sum of product form

- The diagram given below represents __________

► Demorgans law

► Associative law

► Product of sum form

► Sum of product form

- The output of an AND gate is one when _______

► All of the inputs are one

► Any of the input is one

► Any of the input is zero

► All the inputs are zero

- The 4-variable Karnaugh Map (K-Map) has _______ cells for min or max terms

► 4

► 8

► 12

► 16 )

- A BCD to 7-Segment decoder has

► 3 inputs and 7 outputs

► 4 inputs and 7 outputs

► 7 inputs and 3 outputs

► 7 inputs and 4 outputs

- Two 2-input, 4-bit multiplexers 74X157 can be connected to implement a ____ multiplexer.

► 4-input, 8-bit

► 4-input, 16-bit

► 2-input, 8-bit

► 2-input, 4-bit

- The PROM

consists of a fixed non-programmable ____________ Gate array configured as a decoder.

► AND

► OR

► NOT

► XOR

- In ABEL the variable „A‟ is treated separately from variable „a‟

► True

► False

- The ABEL notation equivalent to Boolean expression A+B is:

► A & B

► A ! B

► A # B

► A $ B

L-21

- If an active-HIGH S-R latch has a 0 on the S input and a 1 on the R input and then the R input goes to 0, the latch will be

________.

► SET

► RESET

► Clear

► Invalid

- Demultiplexer has

► Single input and single outputs. ► Multiple inputs and multiple outputs.

► Single input and multiple outputs.

► Multiple inputs and single output.

- Which one is true:

► Power consumption of TTL is higher than of CMOS

► Power consumption of CMOS is higher than of TTL

► Both TTL and CMOS have same power consumption

► Power consumption of both CMOS and TTL depends on no. of gates in the circuit.

- one

The first Least Significant digit in decimal number system has

position 0 and weight equal to 1 position 1 and weight equal to 0 position 1 and weight equal to 10 position 0 and weight equal to 10

- The decimal equivalent of the binary number “10011” is

19 (According to rule)

99

29

None of given options

- In ANSI/IEEE Standard 754 “Mantissa” is represented by 32-bits bits

► 8-bits

► 16-bits

► 32-bits

► 64-bits

- The binary value “11011” is equivalent to

1B (According to rule)

1C

1D

1E

- NOR gate is formed by connecting

OR Gate and then NOT Gate

NOT Gate and then OR Gate

AND Gate and then OR Gate

OR Gate and then AND Gate

- “74ALS” stands for

Advanced Low-frequency Schottky TTL

Advanced Low-dissipation Schottky TTL

Advanced Low-Power Schottky TTL

Advanced Low-propagation Schottky TTL

- An adder circuit can be used to perform subtraction operation

True

False

- For a 3-to-8 decoder how many 2-to-4 decoders will be required?

2)

3

4

1

- 3-to-8 decoder can be used to implement Standard SOP and POS

Boolean expressions

True

False

- Two 2-input, 4-bit multiplexers 74X157 can be connected to implement a multiplexer.

2-input, 4-bit

4-input, 8-bit

4-input, 16-bit

2-input, 8-bit

- The four outputs of two 4-input multiplexers, connected to form a

16-input multiplexer, are connected

together through a 4-input gate

AND

OR

NAND

XOR

- The Programmable Array Logic (PAL) has AND array and a OR array

Fixed, programmable

Programmable, fixed

Fixed, fixed

Programmable, programmable

- Sequential circuits have storage elements

True

False

- Demultiplexer has

Single input and single outputs.

Multiple inputs and multiple outputs.

Single input and multiple outputs.

Multiple inputs and single output.

- Which of the following is a volatile memory?

1. PROM

2.

DRAM

3.

EPROM

4. EEPROM

- ________ is used when the output is connected back to the input of the PAL or if the output pin is used as an input only.

    - Combinational Input/output

    - Combinational Output

    - Combinational Input

    - Programmable polarity

- The AND Gate performs a logical ________ function.

    - Addition

    - Subtraction

    - Multiplication

    - Division

- The Adjacent 1s Detector accepts 4-bit inputs. If ________ adjacent 1s are detected in the input, the output is set to high.

    - 2

    - 4

3.

1

4.

0

- In the keyboard encoder, how many times per second does the ring counter scan the key board?

    - 600 scans/second

    - 625 scans/second

3.

650

scans/second

4.

700

scans/second

- The FAST Model Page Access allows ________ memory read and access times when reading successive data values stored in consecutive locations on the same row.

1. Slow

2.

Faster

3.

Medium

4. Modern

- GAL can be reprogrammed as instead of fuses E2CMOS logic is used which can be programmed to connect a ________ with a ________.

1. column, row

2.

row, column

3.

column, column

4. row, row

- Which of the following Output Equations determines the output of the State Machine?

1. MIN = Q0Q1

2.

MAX = Q0Q1EN

3.

MIN = Q0Q1EN

4. MAX = Q1EN

- The maximum value, represented by a single hexadecimal digit is ________.

1. "E"

2.

"F"

3.

"G"

4. "H"

- If the voltage drop across the active load is 0 volts due to absence of current the comparator output is a ________.

    - 0

    - 1

- The Static Ram (SRAM) is non-volatile and is not a ________ density memory as a latch is required to store a single bit of information.

1. Low

2.

High

3.

Medium

4. Hot

- DE Morgan’s two theorems prove the equivalency of the NAND and ________ gates and the NOR and ________ gates respectively.

    - Negative-OR, Negative-AND

    - Negative-AND, Positive-OR

    - Positive-OR, Negative-AND

    - Positive-OR, Positive-AND

- Two signals ________ and ________ provide the timing inputs to the State Machine.

    - NSSR and EWSR

    - LTIME and STIME

3.

PTIME and QTIME

4.

NSGrn and NSYel

- The 74HC163 is a 4-bit Synchronous counter, it has ________ data output pins.

    - 2

    - 4

    - 6

    - 8

- PLDs have In-System Programming (ISP) capability that allows the ________ to be programmed after they have been installed on a circuit board.

    - PLAs

    - PALs

    - PLDs

    - EPROM

- The CONSTATE.CLK = Clock is used to indicate that the ________ state variables change on a clock transition.

1.

CONSTATE

2.

FLOOR

    - MOTION

    - OPEN

- Two types of memories namely the first in-first out (FIFO) memory and last in first out (LIFO) are implemented using ________.

1.

Shift Registers

2.

Circular Buffers

    - Ring Buffers

    - Reduce Registers

- The normal data inputs to a flip-flop (D, S and R, J and K, T) are referred to as ________ inputs.

    - Sequential

    - Asynchronous

    - Synchronous

    - Combinational

- For a down counter that counts from (111 to 000), if current state is "101" the next state will be ________.

1. 111 2. 110

    - 010

    - None of the given

- The ________ gate and ________ gate implementation connected at the B input of the 4-bit Adder is used to allow Complemented or Un-

Complemented B input to be connected to the Adder input.

    - AND, NOR

    - AND, NOT

    - AND, OR

    - XOR, NAND

- The Synchronous SRAM also has a Burst feature which allows the Synchronous SRAM to read or write up to ________ location(s) using a single address.

    - One

    - Two

    - Three

    - Four

- In NAND based S-R latch, output of each ________ gate is connected to the input of the other ________ gate.

    - NOR, NAND

    - NAND, NOR

    - NOR, NOR

    - NAND, NAND

- Implementing the Adjacent 1s detector circuit directly from the function table based on the SOP form requires ________ gates for the 8 product terms (minterms) with an 8-input OR gate.

1. 8 OR

2.

8

AND

3.

8

XOR

4. 8 NOR

- 8-bit parallel data can be converted into serial data by using ________ multiplexer.

1. 4-to-2

2.

8-

to-1

3.

4-

to-4

4. 8-to-4

- The ________ input overrides the ________ input.

    - Asynchronous, synchronous

    - Synchronous, asynchronous

    - Preset input (PRE), Clear input (CLR)

    - Clear input (CLR), Preset input (PRE)

- A SOP expression can be implemented by an ________ combination of gates.

    - OR-XOR

    - AND-NAND

3.

AND-OR

4.

XOR-NOR

- The 64-cell array organized as 8 x 8 cell array is considered

    - as an 64 byte memory

    - as a 16 byte memory

3.

as an 8 byte memory

4.

as an 4 byte memory

- The terminal count of a 4-bit binary counter in the UP mode is ________.

1.

1100

2.

0011

    - 1111

    - 0000

- A 3-variable karnaugh map has

1.

eight cells

2.

three cells

    - sixteen cells

    - four cells

- An Asynchronous Down-counter is implemented (Using J-K flip-flop) by connecting ________.

    - Q output of all flip-flops to clock input of next flip-flops

    - Q' output of all flip-flops to clock input of next flip-flops

    - Q output of all flip-flops to J input of next flip-flops 4. Q' output of all flip-flops to K input of next flip-flops

- Memory is arranged in ________.

    - linear fashion

    - two-dimensional manner

    - three-dimensional manner

    - random fashion

- If two numbers in BCD representation generate an invalid BCD number then the binary ________ is added to the result.

    - 1001

    - 0110

    - 1111

    - 1100

- Subtractors also have output to check if 1 has been ________.

1.

Primed

2.

Shifted

    - Complemented

    - Borrowed

- The Test Vector definition defines the test vectors for all the three counter inputs and ________ counter output/outputs.

    - One

    - Two

3.

Three

4.

Four

- A multiplexer with a register circuit converts

1. Serial data to parallel

2.

Parallel data to serial

3.

Serial data to serial

4. Parallel data to parallel

- A decade counter can be implemented by truncating the counting sequence of a MOD-20 counter.

1.

True

2.

False

- The n flip-flops store ________ states.

1. 1

2.

2

^n

3.

0

4. 2^(n+1)

- The S-R latch has two inputs, therefore ________ different combinations of inputs can be applied to control the operation of the S-R latch.

1. two

2.

four

3.

eight

4. sixteen

- Why demultiplexer is called a data distributor?

1. The input will be distributed to one of the outputs 2. The input will be selected for the output

    - The output will be distributed to one of the inputs

    - Single input to Single Output

- When the transmission line is idle in an asynchronous transmission

    - It is set to logic low

    - It is set to logic high

    - It remains in previous state

    - State of transmission line is not used to start transmission

- UVERPROM is stands for

    - Ultra-Variant

    - Ultra-Vibrant

3.

Ultra-Voilet

4.

Ultra-Visible

- In memory write cycle, the time for which the WE signal remains active is known as the ________.

    - Write address setup

    - Write pulse width

GROUP

    - Write delay width

    - Write data time

- The outputs of SR latches in elevator state machine are feed back to the ________ gate array for connection to the D-flipflops.

1. NOT

2.

AND

3.

OR

4. XOR

- PALs tend to execute ________ logic.

1. SPD

2.

SOP

3.

SAC

4. SAP

- The ROM used by a computer is relatively ________ as it stores few byres of code used to Boot the Computer system on power up.

1.

Small

2.

Large

    - Heavy

    - High

- Which signal must remain valid in memory write cycle after data is applied at the data input lines and must remain valid for a minimum time duration tWD?

1. -CS

2.

-

WE

3.

W

4. OE

- You have to choose suitable option when your timer will reset by considering this given code:

TRSTATE.CLK = clk;

TMRST: = (TRSTATE = = NSY2) # (TRSTATE = = EWY2);

1.

NSY2 or EWY2

2.

NSSR or TMRST

    - EWSR or NSRED

    - EWRed or EWYel

- A NOR based S-R latch is implemented using ________ gates instead of ________ gates.

    - XOR, NAND

    - NOR, XOR

3.

NOR, NAND

4.

OR, XOR

- Implementation of Latch is required almost ________ transistor.

    - Two

    - Four

3.

Six

4.

Eight

- In distributed mode, for a 1024 x 1024 DRAM memory and a refresh cycle of 8 msec, each of the 1024 rows has to be refreshed in ________ when Distributed refresh is used.

1. 4.8 microsec 2. 5.9 microsec

3.

7.8

microsec

4.

5.5

microsec

- The NOR logic gate is the same as the operation of the ________ gate with an inverter connected to the output.

1. AND

2.

NAND

3.

OR

4. NOT

- For a Standard SOP expression, a ________ is placed in the cell corresponding to the product term (Minterm) present in the expression.

1. 0

2.

1

3.

x (don't care condition)

4. Any of given option depending on SOP term

- Select the mode of programming in which GAL16V8 can be programmed:

    - Simple Mode

    - Complex Mode

    - Registered Mode

    - All of the given

- Divide-by-32 counter can be achieved by using

    - Flip-Flop and DIV 10

    - Flip-Flop and DIV 16

3.

Flip-Flop and DIV 32

4.

DIV 16 and DIV 32

- The next state table for REQ1, FLOOR1 and OPEN inputs indicates that the ________ can be pressed at any time either on the first floor or the second floor in elevator.

    - REQ0

    - OPEN

    - REQ1 	 	AL-JUNAID INSTITUTE OF GROUP

    - FLOOR1

- Consider A=1, B=0, C=1. A, B and C represent the input of three bit NAND gate, the output of the NAND gate will be ________.

1. Zero

2.

One

3.

Undefined

4. No output as input is invalid

- A 4-bit binary up/down counter is in the binary state of zero. The next state in the DOWN mode is:

1. 0001 2. 1000 3. 1110

4. 1111

- Adding two octal numbers "36" and "71" result in ________.

    - 213

    - 123

3.

127

4.

345

- The ABEL Input file can use a ________ instead of the equation to specify the Boolean expressions.

1.

Truth Table

2.

State Diagram

    - Karnaugh Map

    - Logic Circuit

- The domain of the expression AB'CD + AB' + C'D +

B is

    - A and D

    - B only

    - A, B, C and D

    - None of the given

- If the number of samples that are collected is reduced by half, the reconstructed signal will be ________ from/to the original.

1. Different

2.

Same

3.

Equal

4. Opposite

- In DRAM read cycle R /W- signal is activated to read data which is made available on the ________ data line.

1. D(IN)

2.

D(OUT)

3.

D(AB)

4. D(INT)

- In case of cascading Integrated Circuit counters, the enable inputs and RCO of the Integrated Circuit counters allow cascading of multiple counters together.

1.

True

2.

False

- Implementation of the FIFO buffer in ________ is usually takes the form of a circular buffer.

1.

RAM AL-JUNAID INSTITUTE OF GROUP

2.

ROM

    - PPROM

    - Flash Memory

- As data values are written or read from the RAM Stack Pointer Register increments or decrements its contents always pointing to the stack ________.

1. Bottom

2.

Top

3.

Down

4. Vertex

- Which one flip-flop has an invalid output state?

    - T

    - JK

3.

SR

4.

D

- The output of a NAND gate is ________ when all the inputs are one.

1.

Zero

2.

One

    - Available

    - Not available

- The Transition table is very similar to the ________ table.

1. Truth

2.

State

3.

Transition

4. None of the given

- Consider the sum of weight method for converting decimal into binary value, ________ is the highest weight for 411.

    - 64

    - 128

3.

256

4.

512

- Canonical form is a unique way of representing ________.

      - SOP

      - Minterm

      - Boolean Expression

      - POS

- ________ Counters as the name indicates are not triggered simultaneously.

1. Asynchronous

2.

Synchronous

3.

Positive-Edge triggered

4. Negative-Edge triggered

- Cin is part of ________ Adder.

1. Half

2.

Full

3.

Single

4. Double

- Flash memory Operation are classified into ________ different operation.

1.

Two

2.

Three

      - Four

      - Five

  - A Product term is 0 when______

      - Any one literal is 0

      - Any of the literals is 1

      - At least two literals are 1

      - All the literals are 1

  - In 8-inpit multiplexer, the two outputs are connected through a/an___gate.

a. AND

5. OR 	 	AL-JUNAID INSTITUTE OF GROUP

      - NOT

      - NOR

  - ____ Device dissipate varying amount of power depending upon the frequency of operation.

      - TTL

      - CMOS

      - Storage

      - Peripheral

  - Boolean Addition operation is performed by a(an)___ gate.

      - AND

      - OR

      - XOR

      - NAND

  - A SOP expression can be implemented by an_____ combination of gates.

      - OR-XOR

      - AND-AND

      - AND-OR

      - XOR-NOR

  - The maximum decimal number that can be represented using the 64-bit unsigned representation is _______.

      - 2^63

      - (2^64)-1

      - (2^64+

      - 2^64

  - In 16-bit ALU, The G output is activated if the 4-bi unit generate a Carry ____ irrespective of Carry___.

      - In,In

      - In,Out

      - Out,In

      - Out,Out

  - A standard POS from has ____ ters that have all the variables in the domain of the expression.

      - Sum

      - Product

      - Min

      - Composite

  - In Cascading Priority Encoders, the EO output is connected to the EI input of the encoder which handles_____.

      - Lower priority outputs

      - Higher priority outputs

      - Lower priority inputs

      - Higher priority inputs

  - Which of the following is the example of comparater?

      - OR

      - AND

      - XOR

      - XNOR

  - IN CMOS 5 Volt series, Input voltage of Logic high signal (VIH) with a ranges from ____ to ____ volts.

      - 4,5,5

      - 0 ,5

      - 0,3,5

      - 3,5,5

  - The Adjacent 1 S Detector accepts 4-bits input. If ____ adjacent 1S are detected in the input, the output is set to high.

      - 2

      - 4

      - 1

      - 0

  - DE Morgan’s two theorems prove the equivalency of the NAND and

_____ gates and the NOR and _____ gates respectively.

      - Negative-OR, Negative-AND

      - Negative-AND, Negative-OR

      - positive-OR, Negative-AND

      - positive-AND, Negative-OR

  - Adding two octal numbers “36 and 71” result in ____.

      - 213

      - 123

      - 127

      - 345

  - Any of the _____ forms of the Karnaugh Map can be used to simplify

Boolean expressions

      - Three

      - Four

      - Two

      - Five

  - Quine-McCluskey and K-Map methods are used for _____ of Boolean expression.

      - Multiplication

      - Addition

6. Simplification 	 	AL-JUNAID INSTITUTE OF GROUP

c. Subtraction

  - The number “1259” may belong to ____ number system.

      - Binary number system

      - Octal or Decimal system

      - Decimal or Hexadecimal system

      - Binary or Hexadecimal system

  - The series of TTL chips are characterized by their____.

      - Switching Speed only

      - Power Dissipation only

      - Both

      - None of these

  - All ABEL statements must end with_____.

      - :

      - ,

      - ;

      - “

  - In sequential circuit memory elements are connected with____.

      - Logical circuit

      - Clock

      - Feedback path

      - External Event

  - In the 32-bit Single Precision Floating Point format, the exponent value____ is reserved to represent infinity exponents.

      - 98

      - 99

      - 255

      - 256

  - The _____ output has the output of the OR gate connected through an

XOR gate to the tri-state buffer.

      - Combinational

      - Combinational input/

      - PLA

      - None

  - The limitation in implementation of parallel binary address is known as________.

      - Delay

      - Carry propagation

      - Carry input

      - Carry output

  - The Gray code is different form the unsigned binary code

because___.

      - Successive value of Gray code by only one bit

      - Gray code is positional code

      - Gray code not support negative values

      - Gray code ranges from “o” to “9”

  - Removing the NOT gate at the output of the NOR gate result in an____.

7. OR gate 	 	 	AL-JUNAID INSTITUTE OF GROUP

      - NAND gate

      - AND gate

      - NOT gate

  - Portable devices that run on batteries use___ circuit that have low power dissipation.

      - Series

      - Integrated

      - Parallel

      - Electric

  - The domain of the expression AB’CD+B is

      - A and D

      - A,B,C and D

      - C and D only

      - B only

  - _____ is a single input gate

      - AND

      - OR

      - NOT

      - XOR

29.To represent in digital value, the number of digit (0s and 1s) that represent a quantity is ____ to the range of values that are to be represented.

      - Equal

      - Greater

      - Lesser

      - Not equal

30. 	BCD code of 16 is____.

      - 10001

      - 00010001

      - 00010111

      - 01110001

31. To determine the seven expressions for each of the seven outputs in 7-segment display, seven____ variable Karnaugh Maps are used.

8. 3 	 	AL-JUNAID INSTITUTE OF GROUP

      - 4

      - 5

      - 2

32. In Odd parity generator circuit which gate is used to detect parity errors?

      - OR

      - NOR

      - XOR

      - AND

33. A 3-variable Karnaugh map has

      - Eight cells

      - Three cells

      - Sixteen cells

      - Four cells

34. The measurable values generally change over a

      - Specified period

      - Discrete range

      - Time

      - Continuous range

35. ____ uses E2CMOS technology which is Electrically Erasable CMOS

instead of Bipolar technology and fusible links.

      - PAL

      - GAL

      - PLA

      - PROM

36. When the number 29 is represent on 7-segment display, which BCD input is represented on LSD display unit?

      - 1000

      - 1001

      - 1010

      - 1100

37. How many of enable inputs is(are) active-low in 74xx138 3 to 8 Decorder?

      - One

      - Two

      - Three

      - Four

38.The simplified expression using either of the two K-maps are_____.

      - Similar

      - Different

      - Identical

      - None-identical

39. Which of the following expression in the product of sums form?

      - (A + B)(C+D)

      - (AB)(CD)

AL-JUNAID INSTITUTE OF GROUP

      - AB(CD)

      - AB+CD

40. CMOS technology is characterized by low power dissipation with____ switching speeds.

      - Slow

      - Fast

      - Average

      - Medium

41. GAL Two 2-bit comparator circuits can be connected to form single 4-bit

comparator

    - True

    - False

42. High level Noise Margins (VNH) of CMOS 5 volt series circuits is

_____________

      - 0.2 V

      - 0.5 V

      - 0.9 V

      - 3.3 V

43. The output of the expression F=A+B+C will be Logic ________

when A=0, B=1, C=1. the symbol‟+‟ here represents OR Gate

      - Undefined

      - One

      - Zero

      - 10(binary)

44. If an active-HIGH S-R latch has a 0 on the S input and a 1 on the R input and then the R input goes to 0, the latch will be ________.

      - SET

      - RESET

      - Clear

      - Invalid

45. 3.3 v CMOS series is characterized by __________ and

_________as compared to the 5 v CMOS series.

      - Low switching speeds, high power dissipation

      - Fast switching speeds, high power dissipation

      - Fast switching speeds, very low power dissipation (page61)

      - Low switching speeds, very low power dissipation

46. The binary value “1010110” is equivalent to decimal __________

      - 86

      - 87

      - 88

      - 89

47. The _______ Encoder is used as a keypad encoder.

      - 2-to-8 encoder

      - 4-to-16 encoder

      - BCD-to-Decimal

      - Decimal-to-BCD Priority

48.How many data select lines are required for selecting eight inputs?

      - 1

      - 2

      - 3

      - 4

49. The Quad Multiplexer has _____ outputs

      - 4

      - 8

      - 12

      - 16

50. 	Demultiplexer has

      - Single input and single outputs.

      - Multiple inputs and multiple outputs.

      - Single input and multiple outputs.

      - Multiple inputs and single output.

51. The expression _________ is an example of Commutative Law for Multiplication.

      - AB+C = A+BC

      - A(B+C) = B(A+C)

      - AB=BA

      - A+B=B+A

52. The look-ahead carry circuits_______

      - Add a 1 to complemented inputs

      - Reduce propagation delay

      - Increase ripple delay

      - Determine sign and magnitude

53. What is the output expression of segment ‘b’ implementation in BCD to 7segment decoder?

      - B+C’D’+CD

      - B’+C’D’+CD

      - B+C’D’+C’D

      - B’+C’D’+CD’

54. 2-input, 8-bit Multiplexer, by setting the S input to logic_____

the____ inputs of both the multiplexers are selected.

      - Low

      - High, B

      - Low, C

      - Low, C

55. The maximum decimal number that can be represented using the

64-bit unsigned representation__________

      - 2^63

      - (2^64)-1

      - (2^64)+1

      - 2^64

56. When two or more products terms are assumed by Boolean addition, the result is a ____

      - POS

      - SOP

      - Boolean

      - Simplified

57. Tri-State Buffer is a ___ gate with a control line that disconnects the

      - OR

      - AND

      - NAND

      - NOT

58.The 4-bits 2’s complement representation of “7” is ____________

      - 0111

      - 1111

      - 1001

      - 0110

59. ________and _________ are the steps of the Quine-McCluskey.

      - Draw a table and make groups

      - Write binary codes and write SOP

      - Find prime implicants and select minimal set of the prime implicants.

      - None of the given

60. The binary number 1011,101 has an Integer part represented by_______ and a fraction part ___ separated by a decimal point.

      - 1011,101

      - 101,1011

      - 101,1101

      - 10111,11

61. Subtractors also have output to check if 1 has been _________

      - Primed

      - Shifted

      - Complemented

      - Borrowed

62.CMOS technology is characterized by low power dissipation with______ switching speeds.

      - Slow

      - Fast

      - Average

      - Medium

63. The __________ description is used to simulate the logic circuit

and verify its operation.

      - Test file

      - Declaration

      - Logic

      - Test vector

64. How many outputs can an integrated circuit comparator have?

      - One

      - Two

      - Three

      - Four

65. Which of the following is not the correct method of grouping?

      - Along adjacent columns

      - On comers

      - Diagonally

      - Along adjacent rows

66. 	The output of the expression F=A.B.C will be logic ______

when A=1,B=0, C=1.

      - Undefined

      - One

      - Zero

      - No output as input is valid

67. The _____ gate and ___ gate implementation connected at the B

input of the 4-bit Adder is used to allow complemented or UnComplemented B input to be connected to the Adder input.

      - AND,NOR

      - AND,NOT

      - AND,OR

      - XOR,NAND

68. In the 32-bit Single Precision Floating point format, the exponent value _____ is reserved infinity exponent.

      - 98

      - 99

      - 255

      - 256

69. 	The Boolean expression (AB’CS’) is used

      - A sum term

      - A product term

      - A literal term

      - A max term

70. 	The product of an XOR gate is zero(0), when ________

    - All the inputs are zero

    - Any of the inputs is zero

    - An of the inputs is one IV.

All the inputs are one

        - I only

        - IV only

        - I and IV only

        - II and III

71. ________ methods are used to Convert Decimal fractions to Binary.

      - 1

      - 2

      - 3

      - 4

72. To display the number___ the BCD number 0010 representing the MSD is applied at the inputs of the BCD to 7-segment display circuit connected to the MSD &-Segment Display digit

      - 19

      - 29

      - 39

      - 49

73. The look-ahead carry circuits_________

      - Add a 1 to complemented inputs

      - Reduce propagation delay

      - Increase ripple delay

      - Determine sign and magnitude.

- If two numbers in BCD representation generate an invalid BCD number then the binary __ is added to the result

      - 1001

      - 0110

      - 1111

      - 1100

- Both the multiplexers are selected simultaneously when ________

is set to logic ______ in 2-inputs, 8-bit Multiplexer.

      - G, Low

      - G, High

      - S, Slow

      - S, High

- Function labels required to represent the input/output combinations for each segment in 7-segment display

      - 5

      - 7

      - 8

      - 10

- Multiplexers are also known as __________

      - Date distributors

      - Data selectors

      - Data manipulators

      - Data setters

- The PLA can be programmed to give and output of constant

______ or _______

      - 0,1

      - 1,2

      - 2,3

      - 0,0

- Cin is part of ________ Adder.

      - Half

      - Full

      - Single

      - Double

- The look-ahead carry circuits__________

      - Add 1 to complemented inputs

      - Reduce propagation delay

      - Increase ripple delay

      - Determine sign and magnitude

- Which of the following gates has the outputs 1 if and only if at last one input is 1?

          - AND

          - NOR

          - NAND

          - OR

- A sop expression can be implemented by on ___

combination of gates.

      - OR-XOR

      - AND-NAND

      - AND-OR

      - XOR-NOR

83.The carry, instead of rippling through the 4-bits of the individual ALU circuit, has to propagate through ___ ALU units in 16-bit ALU.

      - Two

      - Four

d. 9

91. Digital circuits operates with______ voltage value(s)

      - 1

      - 2

      - 3

      - 4

92.Which of the following is the octal equivalent of 28 decimal numbers?

      - 31

      - 32

      - 33

      - 34

93. In cascading Priority Encoders, the EO output is connected to the EI of the encoder which handles__________

      - Lower priority outputs

      - Higher priority outputs

      - Lower priority inputs

      - Higher priority inputs

94. To determine the seven expressions for each of the seven outputs in

7-segment display, seven _____ variable Karnaugh maps are used.

      - 3

      - 4

      - 5

      - 2

95. The output of a NAND Gate is ____ when all the inputs are one.

      - Zero

      - One

      - Available

      - Not available

96. 	The _______ is the slowest and consumes more power.

      - Standard TTL

      - Schottky TTL

      - Advanced Schokottky TTL

      - Low-Power Schottky TTL

97. The between expression X-AB+CD represents

      - Two Ors ANDed together

      - A 4-input AND gate

      - Two ANDs ORed together

      - An eclusive-OR

98. The expression F-A+B+C describes the operation of three bits___ Gate.

      - OR

      - AND

      - NOT

      - NAND

99. Which one of the following is NOT a valid rule of Boolean Algebra?

      - A+1=1

      - A=A’

      - AA=A

      - A+0=A

100. 	A 5-Variable Karnaugh map has

      - Sixteen cells

      - Thirty two cells

      - Sixty four cells

      - None of these

AL-JUNAID INSTITUTE OF GROUP

101. ____ is invalid number of cells in a single group formed by the adjacent cells K-map

      - 2

      - 8

      - 12

      - 16

102. 	In 32-bit Single –Precision floating point format representation the range of exponent value is from _____to ______

      - +127 to -126

      - +127 to -254

      - +128 to -254

      - +256 to -255

103. 	____ has the fastest switching speed and low power requirements

      - Advanced low power schottky

      - Fast TTL

      - Standard TTL

      - Schottky TTL