Sample Paper

FINALTERM  EXAMINATION

Fall 2022

CS302 – Digital Logic Design

Time: 90 min

Marks: 60

Question No: 1     (Marks: 01) - Please choose the correct option

Rise and fall in temperature during 24 hours is an example of a?

- Continuous Quantity

- Digital Quantity

- Discrete Quantity

- Digital & Discrete both

Question No: 2     (Marks: 01) - Please choose the correct option

The Sum-of-Weights method can be used to convert a Binary number of any magnitude to its equivalent.

- Hexadecimal representation

- Octal representation

- Decimal representation

- Binary representation

Question No: 3    (Marks: 01) - Please choose the correct option

In Binary to Hexadecimal Conversion, the Binary string is divided into small groups of ___________ starting from the least significant bit.

- 1-bits

- 2-bits

- 3-bits

- 4-bits

Question No: 4    (Marks: 01) - Please choose the correct option

F in Hexadecimal is equivalent to ___________ in binary.

- 1111

- 1011

- 0011

- 0000

Question No: 5    (Marks: 01) - Please choose the correct option

The Boolean expression of AND gate is ___________

- Y = A.B

- Y = A+B

- Y = A-B

- Y = A/B

Question No: 6    (Marks: 01) - Please choose the correct option

The logic NOR function is the combination of two logic functions that are ___________logics.

- OR & NOT

- AND & NOT

- OR & AND

- NOT & XOR

Question No: 7    (Marks: 01) - Please choose the correct option

The output of a NAND is ___________ when all inputs to the NAND gate are 1s.

- 1

- 00

- 10

- 0

Question No: 8    (Marks: 01) - Please choose the correct option

Demorgan’s First Theorem states that the complement of a ___________ of variables is equal to the sum of the complements of the variables.

- Sum

- Subtraction

- Product

- Division

Question No: 9    (Marks: 01) - Please choose the correct option

The simplified Boolean expression of this logic circuit is:

- F = A.B.C.D

- F = A+B+C+D

- F = ̅A.̅B.C.D

- F = A.B.C+D

Question No: 10    (Marks: 01) - Please choose the correct option

A 4-variable K-Map has an array of ___________ cells

- 8

- 16

- 24

- 32

Question No: 11    (Marks: 01) - Please choose the correct option

A K-map for ___________ variables can be constructed by using two 4-variable K-maps.

- 5

- 8

- 15

- 30

Question No: 12    (Marks: 01) - Please choose the correct option

A standard way to express a Boolean expression is the ___________   form.

- POS

- SOS

- POP

- SOP

Question No: 13    (Marks: 01) - Please choose the correct option

XOR and ___________   gates are used to implement the Odd-Parity Generator Circuit

- OR

- XNOR

- NOR

- NAND

Question No: 14    (Marks: 01) - Please choose the correct option

The 2’s complement of any number is obtained by taking the 1’s complement of a number and then ___________  a 1 to the 1’s complement.

- Adding

- Subtracting

- Multiplying

- Dividing

Question No: 15    (Marks: 01) - Please choose the correct option

The basic function of a Comparator is to ___________ two binary quantities.

- Multiply & Divide

- Compare

- Subtract

- Add

Question No: 16    (Marks: 01) - Please choose the correct option

The Decimal-to-BCD Encoder has ___________ inputs, for the decimal digits 0 to 9 and four outputs corresponding to the 4-bit BCD output.

- Two

- Eight

- Ten

- Sixteen

Question No: 17    (Marks: 01) - Please choose the correct option

Demultiplexer has a single input and ___________ outputs.

- Two

- Three

- Four

- Several

Question No: 18    (Marks: 01) - Please choose the correct option

In this figure of Standard PAL Numbering, 8 at the last indicates

- Eight Inputs

- Eight Outputs

- Eight bits

- Eight Decimals

Question No: 19    (Marks: 01) - Please choose the correct option

ABEL Symbol for Logic operation OR is

- !

- &

- #

- $

Question No: 20    (Marks: 01) - Please choose the correct option

A.B in boolean notation is equivalent to ___________ in ABEL notation.

- !A

- A&B

- A#B

- A$B

Question No: 21      (Marks: 01) - Please choose the correct option

Why latches are called memory devices?

- It can store 8 bits of data

- It has an internal memory of 4 bit

- It can store one bit of data

- It can store an infinite amount of data

Question No: 22      (Marks: 01) - Please choose the correct option

Does the NAND latch work when both inputs are ___________?

- 1

- 0

- Inverted

- Don’t care

Question No: 23       (Marks: 01) - Please choose the correct option

The truth table for an S-R flip-flop has how many VALID entries?

- 1

- 2

- 3

- 4

Question No: 24       (Marks: 01) - Please choose the correct option

Does the basic latch consist of ___________?

- Two inverters

- Two comparators

- Two amplifiers

- Two adders

Question No: 25       (Marks: 01) - Please choose the correct option

In D flip-flop, D stands for _____________?

- Distant

- Data

- Desired

- Delay

Question No: 26       (Marks: 01) - Please choose the correct option

How many stable states do combinational circuits have?

- 3

- 4

- 5

- 2

Question No: 27       (Marks: 01) - Please choose the correct option

In the serial shifting method, data shifting occurs ____________?

- One bit at a time

- Two-bit at a time

- Four bit at a time

- simultaneously

Question No: 28       (Marks: 01) - Please choose the correct option

The full form of SIPO is _________________?

- Serial-in Parallel-out

- Parallel-in Serial-out

- Serial-in Serial-out

- Serial-In Peripheral-Out

Question No: 29       (Marks: 01) - Please choose the correct option

The group of bits 10110111 is serially shifted (right-most bit first) into an 8-bit parallel output shift register with an initial state of 11110000. After two clock pulses, the register contains ______________?

- 10111000

- 10110111

- 11110000

- 11111100

Question No: 30       (Marks: 01) - Please choose the correct option

What type of register would have a complete binary number shifted in one bit at a time and have all the stored bits shifted out one at a time?

- Parallel-in Parallel-out

- Parallel-in Serial-out

- Serial-in Serial-out

- Serial-in Parallel-out

Question No: 31       (Marks: 01) - Please choose the correct option

What is the function of a buffer circuit?

- To provide an output that is inverted from that of the input

- To provide an output that is equal to its input

- To clean up the input

- To clean up the output

Question No: 32       (Marks: 01) - Please choose the correct option

Which of the following has the lowest access time?

- RAM

- ROM

- Flag

- Registers

Question No: 33       (Marks: 01) - Please choose the correct option

Logic circuits can also be designed using _________?

- RAM

- ROM

- PLA

- PLD

Question No: 34       (Marks: 01) - Please choose the correct option

If enable input is high, then the multiplexer is _________?

- Disable

- Enable

- High Impedance

- Saturation

Question No: 35       (Marks: 01) - Please choose the correct option

A shift register is defined as _________?

- The register is capable of shifting information either to the right or to the left

- The register is capable of shifting information to another register

- The register is capable of shifting information to the left only

- The register is capable of shifting information to the right only

Question No: 36       (Marks: 01) - Please choose the correct option

Which circuit is generated from D flip-flop due to the addition of an inverter by causing the reduction in the number of inputs?

- Gated D-latch

- Gated T-latch

- Gated JK-latch

- Gated SR-latch

Question No: 37       (Marks: 01) - Please choose the correct option

When a high is applied to the set line of an SR latch, then________?

- Q output goes high

- Q output goes low

- Q` output goes high

- Both Q and Q` go high

Question No: 38       (Marks: 01) - Please choose the correct option

D flip-flop is a circuit having ________?

- 5 NAND gates

- 4 NAND gates

- 3 NAND gates

- 2 NAND gates

Question No: 39      (Marks: 01) - Please choose the correct option

What is a state diagram?

- It provides the same information as the state table

- It is similar to the characteristic equation

- It provides a graphical representation of states

- It is the same as the truth table

Question No: 40      (Marks: 01) - Please choose the correct option

How many different states do a 3-bit asynchronous down counter have?

- 4

- 6

- 8

- 2

Question No: 41      (Marks: 03)

Convert the following hexadecimal number into an equivalent octal number. Also, write down all the steps.

(A10)16

Question No: 42      (Marks: 03)

Convert the following binary number into an equivalent decimal number. Also, write down all the steps.

(1111111)2

Question No: 43      (Marks: 03)

Write the simplified expression for the following table:

Question No: 44      (Marks: 03)

Consider a memory can store 32K nibbles then how many bytes a memory can store? Justify your answer with mathematical proof.

Question No: 45      (Marks: 03)

Apply the shift right operation on the given below 4-bit input and write the output. Consider the 3 clock pulses and write the steps for each pulse.

A = 1011

Question No: 46      (Marks: 03)

Consider the state of the stack given below:

5

2

3

12

Consider the following actions that have been executed and write the final state of the stack.

push 7

pop

pop

Question No: 47      (Marks: 05)

Make the function table for segment ‘d’ for the 7-segment display.

Question No: 48      (Marks: 05)

Consider the 2 to 4 decoder and complete the following function table.

| Input |  | Output |  |  |  |
| --- | --- | --- | --- | --- | --- |
| I1 | I0 | O0 | O1 | O2 | O3 |
| 0 | 0 |  |  |  |  |
| 0 | 1 |  |  |  |  |
| 1 | 0 |  |  |  |  |
| 1 | 1 |  |  |  |  |

Question No: 49      (Marks: 05)

Consider the following Boolean expression and write the equivalent ABEL expression.

F = AB’C + ABC + BD + CD + A’B’C’

Consider the Gated S-R Latch and complete the following table

| Input |  |  | Output |
| --- | --- | --- | --- |
| EN | S | R |  |
| 0 | x | x |  |
| 1 | 0 | 0 |  |
| 1 | 0 | 1 |  |
| 1 | 1 | 0 |  |
| 1 | 1 | 1 |  |

Question No: 50      (Marks: 05)

Consider the gated S-R latch and draw the output of the following timing diagram