FRAME FORMATS

Ethernet Packet Frame

- Preamble allows the receiver to synchronize with signal

- Frame must contain at least 46 bytes to detect collision

- 802.3 standard substitutes length with type field

  - Type field (demux key) is the first thing in data portion

  - A device can accept both frames: type > 1500

Token Ring Frame Format

- “Illegal” Manchester codes in the start and end delimiters

- Frame priority and reservation bits in access control byte

- Demux key in frame control byte

- A and C bits for reliable delivery, in status byte

FDDI Frame Format

- 4B/5B control symbols for start and end of frame

- Control Field

  - 1st bit: asynchronous (0) versus synchronous (1) data

  - 2nd bit: 16-bit (0) versus 48-bit (1) addresses

  - Last 6 bits: demux key (includes reserved patterns for token and claim frame)

- Status Field

  - From receiver back to sender; error in frame

  - Recognized address; accepted frame (flow control)

Frame Format for 802.11(Wireless)

- Control field contains three subfields:

  - 6-bit Type field (data, RTS, CTS, scanning);

  - 1-bit ToDS; and

  - 1-bit FromDS

- A single frame contains up to 2312 bytes of data

ATM Cell Format

- User-Network Interface (UNI)

  - Host-to-switch format

GFC: Generic Flow Control

VCI/VPI: Virtual Circuit/Path Identifier

Type: management, congestion control, AAL5

  - CLP: Cell Loss Priority

  - HEC: Header Error Check (CRC-8)

Network-Network Interface (NNI)

  - Switch-to-switch format

  - GFC becomes part of VPI field

AAL 3/4

- Provides information to allow variable size packets to be sent in fixed-size ATM cells

- Convergence Sublayer Protocol Data Unit (CS-PDU)

  - CPI: Common Part Indicator (version field)

  - Btag/Etag:beginning and ending tags (same)

  - BAsize: hint on reassembly buffer space to allocate

  - Length: size of whole PDU

- Segmented into cells: header/trailer + 44-byte data

ATM Cell Format for AAL 3/4

- Type (is-start? and is-end? bits)

  - BOM (10): beginning of message

  - COM (00): continuation of message

  - EOM (01): end of message

  - SSM (11): single-segment message

- SEQ: sequence number (for cell loss/reordering)

- MID: multiplexing ID (mux onto virtual circuits)

- Length: number of bytes of PDU in this cell

AAL 5  CS-PDU

  - Pad so trailer always falls at the end of ATM cell

  - Length: size of PDU (data only)

CRC-32 (detects missing or misordered cells)

ARP Packet Format