CS716: Advanced Computer Networks

Solution of Assignment 1  MS(CS), Spring 2015

Ms140400095

Question No. 1 Calculate the total time required to transfer a 1000-KB ﬁle in the following cases, assuming an RTT of 100 ms, a packet size of 1 KB and an initial 2 × RTT of

“handshaking” before data is sent.

- The bandwidth is 1.5 Mbps, and data packets can be sent continuously.

- The bandwidth is 1.5 Mbps, but after we ﬁnish sending each data packet we must wait one RTT before sending the next.

- The bandwidth is “inﬁnite,” meaning that we take transmit time to be zero, and up to 20 packets can be sent per RTT.

- The bandwidth is inﬁnite, and during the ﬁrst RTT we can send one packet (2 1 − 1 ), during the second RTT we can send two packets (2 2 − 1 ), during the third we can send four (2 3 − 1 ), and so on.

Question No. 2 Suppose a 100-Mbps point-to-point link is being set up between Earth and a new lunar colony. The distance from the moon to Earth is approximately 385,000 km, and data travels over the link at the speed of light—3 × 108 m/s.

- Calculate the minimum RTT for the link.

- Using the RTT as the delay, calculate the delay × bandwidth product for the link.

- What is the signiﬁcance of the delay × bandwidth product computed in (b)?

- A camera on the lunar base takes pictures of Earth and saves them in digital format to disk. Suppose Mission Control on Earth wishes to download the most current image, which is 25 MB. What is the minimum amount of time that will elapse between when the request for the data goes out and the transfer is ﬁnished?

Question No. 3 Suppose a host has a 1-MB ﬁle that is to be sent to another host. The ﬁle takes 1 second of CPU time to compress 50%, or 2 seconds to compress 60%.

- Calculate the bandwidth at which each compression option takes the same total compression + transmission time.

- Explain why latency does not affect your answer.

Question No. 4

- Show the 4B/5B encoding, and the resulting NRZI signal, for the following bit sequence:

1110 0101 0000 0011

- Assuming a framing protocol that uses bit stufﬁng, show the bit sequence transmitted over the link when the frame contains the following bit sequence:

110101111101011111101011111110  Mark the stuffed bits.

Solution:

- First of all we divide the given bits pattern into the groups of 4 bits. This step is called division.

1110   0101   0000  0011

- The 4 bits are selected in the 1st column and their resulting 5 bit pattern is shown in the 2nd column. This step is called the substitution.

| 4-Bit Data Symbol | 5- Bit Code |
| --- | --- |
| 1110 | 11100 |
| 0101 | 01011 |
| 0000 | 11110 |
| 0011 | 10101 |

- The 3rd step in 4B/5B Encoding is the generation of NRZI signal from the resulting 5 bit patterns. This is given below.

Bits                                                  1 1  1  0  0  0  1  0  1  1  1  1  1  1 0 1 0 1  0   1

NRZI