CS716: Advanced Computer Networks

Assignment 2

Ms140400095

MS(CS), Spring 2015

- An ATM network uses a token bucket scheme for traffic shaping. A new token is put into the bucket every 5 µsec. Each token is good for one cell, which contains 48 bytes of data. What is the maximum sustainable data rate?

Answer to Question 1

Available Data:

Token delay = 5 µsec = 5 x 10-6 sec.

Size of one Cell = 48 Bytes = 48 x 8 Bits = 384 Bits.

So Throughput = Size of Data/Time = 384 Bits/5 x 10-6 sec = 76800000 bps.

Or Throughput (Maximum sustainable data rate) = 76.8 x 106 bps = 76.8 Mbps

- Consider the following design problem concerning implementation of virtual-circuit service. If virtual circuits are used internal to the subnet, each data packet must have a 3-byte header and each router must tie up 8 bytes of storage for circuit identification. If datagrams are used internally, 15-byte headers are needed but no router table space is required. Transmission capacity costs 1 cent per 106 bytes, per hop. Very fast router memory can be purchased for 1 cent per byte and is depreciated over two years, assuming a 40-hour business week. The statistically average session runs for 1000 sec, in which time 200 packets are transmitted. The mean packet requires four hops. Which implementation is cheaper, and by how much?

Answer to Question 2

Available data:

Header Size = 3 Bytes = 3 x 8 Bits = 24 Bits

VCI data size = 8 Bytes = 8 x 8 Bits = 64 Bits

Size of internal datagram header = 15 Bytes = 15 x 8 = 120 Bits

Transmission cost = 1 cent per 106 bytes

Or Transmission cost of 106 Bytes = 1 cent

Or Transmission cost of 8 x 106 Bits = 1 cent

Or Transmission cost of 1 Bit = 1/ (8 x 106) cents = 8 x 10-6 cents per hop.

But there are 4 hops

So the Transmission Capacity of 4 hops = 8 x 4 x 10-6 cents = 3.2 x 10-5 cents

Cost of 1 Byte of Memory = 1cent

Or Cost of 8 bits of memory = 1 cent

Or Cost of 1 bit of memory = 1/8 cent

Depreciation = 2 years = 2 x 52 weeks = 104 weeks = 104 x 40 x 7 x 24 hrs = 698880 hrs = 698880 x 60 x 60 sec = 2.5 x 109  sec

Time period of session = 1000 sec

No of packets are transmitted = 200

No: of Hops = 4

So Total Routers = 5

As each Router tie up 8 bytes of storage so there are 5 x 8 = 40 x 8 Bits of storage for 1000 sec.

Now the Requirement of Datagram Header = No of Packets x No of Hops x  320 Bits = 200  x 4 x 320 bits  = 256000 bits = 2.56 x 106 Bits = 2.56 Mbps

We use virtual circuits because they are cost effective.

- For the network given in the following figure, give the datagram forwarding table for each node. The links are labelled with relative costs; your tables should forward each packet via the lowest-cost path to its destination.

Answer to Question 3

According to the above diagram the cost of each link from lowest to highest is as follows:

C to E = 1 (E is the next hop of C)

B to E = 2 (E is the next hop of B)

D to E = 2 (D is the next hop of E)

A to C = 3 (C is the next hop of A)

C to F = 6 (F is the next hop of C)

A to D = 8 (D is the next hop of A)

Datagram forwarding Table of Node A:

| Destination | Next hop | Remarks |
| --- | --- | --- |
| B | C | ACEB has the least cost |
| C | C |  |
| D | C | ACED has the least cost |
| E | C | ACE has the least cost |
| F | C | ACF has the least cost |

Datagram forwarding Table of Node B:

| Destination | Next hop | Remarks |
| --- | --- | --- |
| A | E | BECA has the least cost |
| C | E | BEC has the least cost |
| D | E | BED has the least cost |
| E | E |  |
| F | E | BECF has the least cost |

Datagram forwarding Table of Node C:

| Destination | Next hop | Remarks |
| --- | --- | --- |
| A | A |  |
| B | E | CEB has the least cost |
| D | E | CED has the least cost |
| E | E |  |
| F | F |  |

Datagram forwarding Table of Node D:

| Destination | Next hop | Remarks |
| --- | --- | --- |
| A | E | DECA has the least cost |
| B | E | DEB has the least cost |
| C | E | DEC has the least cost |
| E | E |  |
| F | E | DECF has the least cost |

Datagram forwarding Table of Node E:

| Destination | Next hop | Remarks |
| --- | --- | --- |
| A | C | ECA has the least cost |
| B | B |  |
| C | C |  |
| D | D |  |
| F | C | ECF has the least cost |

Datagram forwarding Table of Node F:

| Destination | Next hop | Remarks |
| --- | --- | --- |
| A | C | FCA has the least coast |
| B | C | FCEB has the least cost |
| C | C |  |
| D | C | FCED has the least cost |
| E | C | FCE has the least cost |

- Suppose a 100-Mbps delayed-release token ring has 10 stations, a ring latency of 30 μs, and an agreed-upon TTRT of 350 μs.

  - How many synchronous frame bytes could each station send, assuming all are allocated the same amount?

  - Assume stations A, B, C are in increasing order on the ring. Due to uniform synchronous traffic, the TRT without asynchronous data is 300 μs. B sends a 200-μs (2.5-Kb) asynchronous frame. What TRT will A, B, and C then sees on their next measurement? Who may transmit such a frame next?

Answer to question 4:

Available Data:

Bandwidth = 100-Mbps

No: of Stations = 10

Ring Latency = 30 μs = 30 x 10-6 sec = 3 x 10-5 sec

TTRT = 350 μs 350 x 10-6 sec = 3.5 x 10-4sec

- As latency is 30 μs so there will be 350-30 = 320 μs = 320 x 10-6 sec = 3.2 x 10-4 sec

Now the bandwidth = 100 Mbps

So Bandwidth x Delay product = 100 Mbps x 3.2 x 10-4sec

= 100 x 106 bps x 3.2 x 10-4 sec

= 108 bps x 3.2 x 10-4sec = 3.2 x 104 bits = 320000 bits =3200/8 = 4Kbytes

Now there are 10 station so there are (4 x 103Bytes)/10 = 400Bytes each.

So 400 synchronous Bytes each station can send.

- In this case the latency between A,B,C does not matter

The node B sends 200-μs (2.5-Kb) asynchronous frame which means B transmits at T = 0  and the timeline resets at T = -300 where the token passes A,B,C

Now at T = 0 the token passes A but B seizes this token.

Now at T = 200 the node B finishes sending and release the token. Now at this point the node C sees the token, so the total TRT of C = 200 + 300 500 which is too big to transmit.

So at T = 500 the token returns to A , B, C

Now the total TRT of A = 500

Similarly the total TRT of B = 500

And the TRT of C = 300

Which indicates that the node C will transmit next

# Simulation Question (10 points)

Consider the following topology. Perform the following simulations in Network Simulator-2 (NS-2).

Each link is a duplex link with 5Mb bandwidth and 10msec delay. Nodes 1 and 2 send UDP data to node 4, each with packet size of 1000 bytes and 500 packets per second. Nodes 1 and 2 start sending data at 0.5 sec and stop at 4.5 sec. Create a trace file for the simulation and execute nam to view the simulation.

- Write tcl script for the above simulation scenario in file ‘assign1.tcl’.

- Node 3 is an intermediate node, why it discards some packets from its queue?

Really     sorry sir I could not managed to handle NS-2. But I am working hard on this