CS716: Advanced Computer Networks

Solution of Assignment 1 MS (CS), Spring 2015

Ms140400095

Question No. 1 Calculate the total time required to transfer a 1000-KB ﬁle in the following cases, assuming an RTT of 100 ms, a packet size of 1 KB and an initial 2 × RTT of

“Handshaking” before data is sent.

- The bandwidth is 1.5 Mbps, and data packets can be sent continuously.

- The bandwidth is 1.5 Mbps, but after we ﬁnish sending each data packet we must wait one RTT before sending the next.

- The bandwidth is “inﬁnite,” meaning that we take transmit time to be zero, and up to 20 packets can be sent per RTT.

- The bandwidth is inﬁnite, and during the ﬁrst RTT we can send one packet (21-1 ), during the second RTT we can send two packets (22-1 ), during the third we can send four (23-1 ), and so on.

Solution:

We are given the following values:

Bandwidth = 1.5Mbps

File Size = 1000KB

Packet Size = 1KB

RTT = 100ms

Handshaking =?

As handshaking = 2 x RTT = 2 x 100 = 200ms = 2001000= .20 sec

Propagation Time = RTT2= 1002=50ms= .05 sec

Similarly Transmission Time = Message SizeBandwidth= 1000KB1.5Mbps= 1000 x 1024 x 8 bits1.5 x 1000000bps=8.1921.5 ps  =5.4613 sec

- The total time  = Handshaking + Transmission Time + Propagation Time = .20 + 5.4613 + .05 = 5.7113 sec

- In this case the number of RTT’s between the 1st packet and 1000th packet = 999RTT’s (99.9 sec)

So total RTT =   5.7113sec + 99.9 sec = 105.6113 sec

- Bandwidth = ∞

Transmit Time = 0 sec

Packet Size 20KB = 2 x 1024 x 8 = 16384 bits

So the total time = Handshaking + Transmission Time + Propagation Time

- As during the 1st RTT we can send one packet (21-1 = 20 = 1), during the 2nd RTT we can send 2 packets (22-1 =  21 = 2), during the 3rd RTT we can send 4 packets (23-1 = 22 = 4), similarly during the Nth RTT we can send 2n-1 packets. When n = 11 (211-1 = 210 = 1024) then we can send all 1000 packets, the latency of the last batch will be 0.5 RTT

So the total time = 2 RTT 11 RTT + 0.5 RTT = 0.2 + 1.1+ 0.5 = 1.35 sec.

Question No. 2 Suppose a 100-Mbps point-to-point link is being set up between Earth and a new lunar colony. The distance from the moon to Earth is approximately 385,000 km, and data travels over the link at the speed of light—3 × 108 m/s.

- Calculate the minimum RTT for the link.

- Using the RTT as the delay, calculate the delay × bandwidth product for the link.

- What is the signiﬁcance of the delay × bandwidth product computed in (b)?

- A camera on the lunar base takes pictures of Earth and saves them in digital format to disk. Suppose Mission Control on Earth wishes to download the most current image, which is 25 MB. What is the minimum amount of time that will elapse between when the request for the data goes out and the transfer is ﬁnished?

Solution:

Bandwidth = 100Mbps = 100 x 106 bps = 1 x 108 bps

Distance = 385000KM = 385000 x 1000 m = 3.85 x 108 m

Propagation Speed = 3 x 108 m/s

- Propagation Time = DistancePropagation Speed = 3.85 x 108 m3 x 108 m/s=1.2833 sec

So RTT = 1.2833 sec

Now the time required for handshaking = 2 x RTT = 2 x 1.2833 sec = 2.56 sec

- Now Delay x Bandwidth = 2.56 sec x 108 bps = 257 Mbits = 32125000 Bytes = 32.125 MB

- The Delay x Bandwidth Product represents the amount of data the sender can send before it would be possible to receive a response.

- Size of Image  =25 MB

In this case we will require at least one RTT before the picture could begin arriving at the earth.

We will assume bandwidth delay only.

As bandwidth delay = 2.56 Sec

And to finish sending  we will require transmission time.

As Transmission time = Message SizeBandwidth=25MB100Mbps = 25B100bps=25 x 8 b100bps=200 b100bps=2 sec.

Now Total Time to transmit until the last bit arrives on the earth = 2 + 2.56 = 4.56 sec.

Question No. 3 Suppose a host has a 1-MB ﬁle that is to be sent to another host. The ﬁle takes 1 second of CPU time to compress 50%, or 2 seconds to compress 60%.

- Calculate the bandwidth at which each compression option takes the same total compression + transmission time.

- Explain why latency does not affect your answer.

Solution:

- Total size = 1MB

Case 1:

After 50% Compression the new size = 50% of 1MB = 0.5MB

So bandwidth after 1 sec = 0.5MB/1 Sec = 0.5MB/sec

Case 2:

After 60% Compression the new size = 60% of 1MB = 0.6MB

Bandwidth in the 2nd case = 0.6MB/2 sec = 0.3MB/sec

- In this case the latency does not affect the answer because it would affect the compression time.

Question No. 4

- Show the 4B/5B encoding, and the resulting NRZI signal, for the following bit sequence:

1110 0101 0000 0011

- Assuming a framing protocol that uses bit stufﬁng, show the bit sequence transmitted over the link when the frame contains the following bit sequence:

110101111101011111101011111110 Mark the stuffed bits.

Solution:

- First of all we divide the given bits pattern into the groups of 4 bits. This step is called division.

1110   0101   0000 0011

- The 4 bits are selected in the 1st column and their resulting 5 bit pattern is shown in the 2nd column. This step is called the substitution.

| 4-Bit Data Symbol | 5- Bit Code |
| --- | --- |
| 1110 | 11100 |
| 0101 | 01011 |
| 0000 | 11110 |
| 0011 | 10101 |

- The 3rd step in 4B/5B Encoding is the generation of NRZI signal from the resulting 5 bit patterns. This is given below.

Bits                                                  1 1  1  0  0  0  1  0  1  1  1  1  1  1 0 1 0 1  0   1

NRZI

- The required bit stuffing of the given bit pattern is given below.

The zero bits are stuffed after the 5 consecutive 1’s. The stuffed bits are underlined and bolded.

1101 0111 1100 1011 1110 1010 1111 1011 0