|  | View Question. |
| --- | --- |

Q: Computer Networks ( Answered ,   0 Comments )

Question.

| Subject: Computer Networks Category: Computers > Internet Asked by: shabach-ga List Price: $15.00 | Posted: 09 Jan 2005 12:22 PST Expires: 08 Feb 2005 12:22 PST Question ID: 454609 |
| --- | --- |

4. Suppose a 100-Mps point-to-point link is being set up between Earth and a new lunar colony. The distance from the moon to earth is approximately 385,000 km, and data travels over the link at the speed of light-3 x 10^8 m/s. a.) Calculate the minimum RTT for the link. b.) Using the RTT as the delay, calculate the delay x bandwidth product for the link. c.) What is the significance of the delay x bandwidth product computed in (b)? d) A camera on the lunar base takes pictures of Earth and saves them in digital format to disk. Suppose Mission Control on Earth wishes to download the most current image, which is 25 MB. What is the minimum amount of time that will elapse between when the request for the data goes out and the transfer is finished?

Answer.

| Subject: Re: Computer Networks Answered By: livioflores-ga on 09 Jan 2005 22:05 PST Rated: |  |
| --- | --- |

Hi!! a.) Calculate the minimum RTT for the link. The minimum RTT is two times of the propagation delay on the link: minimum RTT = 2 * Distance / Speed of propagation = = 2 * 385,000,000 m / (3*10^8 m/s) = = 2.566 s ----------------------------------------------------------- b.) Using the RTT as the delay, calculate the delay x bandwidth product for the link. delay x bandwidth product = 2.566 s * 100 Mbps = = 256.66 Mbits = = 32.08 Mbytes ------------------------------------------------------------- c.) What is the significance of the delay x bandwidth product computed in (b)? The delay x bandwidth product computed in (b) measures the number of bits that can be ?inflight?, i.e., the amount of data the sender can spend up before receiving a response. 256.66 million bits corresponds to how many bits the sender can send before the sender receives an acknowledgement that the receiver receives the first bit. 128.33 million bits corresponds to how many bits the sender can send before the receiver actually receives the first bit. ------------------------------------------------------------ d) A camera on the lunar base takes pictures of Earth and saves them in digital format to disk. Suppose Mission Control on Earth wishes to download the most current image, which is 25 MB. What is the minimum amount of time that will elapse between when the request for the data goes out and the transfer is finished? File size = 25 MB = 25 * 1024 * 1024 bytes = 26,214,400 bytes = = 26,214,400 * 8 b = = 209,715,200 bits transmission time = File Size/Bandwidth = = 209,715,200 bits / 100,000,000 bits/s = = 2.097152 s It would then take a half of RTT for the Mission Control on Earth to make the download request and another half of RTT for the propagation delay for sending the data from the moon to the earth. Then the minimum amount of time is the sum of the transmission time and the two halves of RTT: Minimum amount of time = transmission time + 2*(1/2)*RTT = = 2.097152 s + RTT = = 2.097152 s + 2.566 s = = 4.663152 s -------------------------------------------------------------- I hope this helps you. Again feel free to request for a clarification if you need it. Best regards. livioflores-ga

shabach-ga rated this answer: Hi livioflores-ga, This is just to tell you that your answers are great. look forward to working with you again.